University Network with Multiple Subnets

Design, Implementation and Delivery Record

Project R075458

Cisco Packet Tracer 9.0.1  |  Repository: github.com/chandu242006/CN_proj

Session record - project complete, published 27 August 2026

# 1. Purpose of This Document

This document is the complete record of the working session in which the university campus network project was designed, written, packaged into a git repository and published to GitHub. It captures what was asked for, what was produced, the reasoning behind each significant design decision, the exact state of the work at the end of the session, and what still remains to be done.

It is written to stand alone. Someone who was not present should be able to read this document and understand the network, locate every file, rebuild the simulation, and continue the work.

# 2. Environment

| Item | Value |
| --- | --- |
| Simulator | Cisco Packet Tracer 9.0.1 |
| Install path | C:Program FilesCisco Packet Tracer 9.0.1 |
| Project folder | C:UserssohanProjectsR075458 |
| Operating system | Windows 11 Home Single Language, 10.0.26200 |
| Git | 2.54.0.windows.1 |
| GitHub CLI | gh 2.94.0 |
| Remote repository | https://github.com/chandu242006/CN_proj.git |
| Branch | main |
| Commit author | Chandhra Sekhar (GitHub noreply address) |

The project folder was deliberately moved out of the OneDrive-synced Documents directory during the session. The reasoning is recorded in section 9.

# 3. What Was Requested

The brief asked for a complete, working implementation of a university campus network in Cisco Packet Tracer, delivered as configurations and build steps only. No academic write-up, no abstract, no viva questions.

The functional requirements were:

- Eight departments, each in its own VLAN and its own IP subnet

- A hierarchical topology: core router, distribution switch, one access switch per department

- Inter-VLAN routing, either Router-on-a-Stick or a Layer-3 switch with SVIs

- DHCP with one named pool per VLAN, servers left static

- An internal DNS zone and a working web server with a real portal page

- Security: enable secret, console and VTY passwords, SSH, unused-port shutdown, port security and ACLs, none of which may break DHCP, DNS or HTTP

- Full testing, troubleshooting and a build checklist

- Every device configuration as a separate plain-text file, so the network can be rebuilt by hand if the .pkt will not open

- A git repository that can be pushed to GitHub and worked on from a second machine

# 4. Network Design

## 4.1 Addressing and VLANs

| VLAN | Name | Department | Network | Gateway | Hosts | Method |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | ADMIN | Administration | 192.168.10.0/24 | 192.168.10.1 | 4 PCs | DHCP |
| 20 | CSE | Computer Science | 192.168.20.0/24 | 192.168.20.1 | 6 PCs | DHCP |
| 30 | ECE | Electronics | 192.168.30.0/24 | 192.168.30.1 | 6 PCs | DHCP |
| 40 | MECH | Mechanical | 192.168.40.0/24 | 192.168.40.1 | 4 PCs | DHCP |
| 50 | LIBRARY | Library | 192.168.50.0/24 | 192.168.50.1 | 4 PCs | DHCP |
| 60 | FACULTY | Faculty | 192.168.60.0/24 | 192.168.60.1 | 4 PCs | DHCP |
| 70 | LAB | Student Labs | 192.168.70.0/24 | 192.168.70.1 | 8 PCs | DHCP |
| 80 | SERVERS | Server Room | 192.168.80.0/24 | 192.168.80.1 | 3 servers | Static |
| 99 | MGMT | Switch management | 192.168.99.0/24 | 192.168.99.1 | 9 SVIs | Static |
| 999 | PARKING | Disabled ports | unrouted | none | none | none |

Within every user VLAN, .1 is the router subinterface, .2 to .10 are excluded from DHCP and held for printers and future static hosts, and clients are leased from .11 upward. The server VLAN has no pool at all.

Servers, all static:

- SRV-DNS  192.168.80.10  -  DNS service for the university.local zone

- SRV-WEB  192.168.80.20  -  HTTP and HTTPS, hosts the campus portal page

- SRV-APP  192.168.80.30  -  HTTP application portal plus FTP file service

DNS A records published in the university.local zone: www to .20, dns to .10, portal to .30, ftp to .30, and the bare domain to .20.

## 4.2 Topology

Three tiers. One 2911 router, one 2960 distribution switch, eight 2960 access switches, 36 PCs and 3 servers. Forty-nine devices and forty-eight cables in total.

| Link | From | To | Type | Cable |
| --- | --- | --- | --- | --- |
| Uplink | R1-CORE Gi0/0 | SW-DIST Gi0/1 | 802.1Q trunk | Straight-through |
| Downlink x8 | SW-DIST Fa0/1-8 | Access switch Fa0/24 | 802.1Q trunk | Cross-over |
| Edge x36 | PC FastEthernet0 | Access switch Fa0/1..n | Access | Straight-through |
| Edge x3 | Server FastEthernet0 | SW-SRV Fa0/1-3 | Access | Straight-through |

There are no routed point-to-point links anywhere in the design. In a Router-on-a-Stick topology the router owns every subnet on subinterfaces of a single physical port, so no transit networks and no routing protocol are required.

Switch-to-switch links are FastEthernet on both ends and the router link is Gigabit on both ends. This is deliberate: in Packet Tracer a Gigabit port connected to a FastEthernet port leaves the link permanently down.

# 5. Design Decisions and Their Justification

## 5.1 Router-on-a-Stick rather than a Layer-3 switch

A single 2911 carries nine 802.1Q subinterfaces, Gi0/0.10 through Gi0/0.80 plus Gi0/0.99, each holding the gateway address for one VLAN.

Why this was chosen:

- It is simpler to build and simpler to read than a 3560 with SVIs

- A 3560 needs ip routing enabled manually, which fails silently if forgotten

- DHCP on a 3560 behaves less predictably in Packet Tracer than on a 2911

- The 802.1Q tag becomes visible in Simulation mode, which is the clearest possible demonstration that inter-VLAN routing is doing what the configuration claims

The trade-off, stated honestly:

Every inter-VLAN packet crosses Gi0/0 twice, once inbound and once outbound. This is the router-on-a-stick hairpin. At forty-nine devices it is nowhere near the link limit; at several thousand users it would be the wrong answer and a Layer-3 switch would be correct. The SVI alternative is written out in full in docs/portability-notes.md section 5.

## 5.2 One addition to the specified addressing plan

The brief specified eight VLANs. Two more were added and the reason is technical, not cosmetic.

VLAN 99, 192.168.99.0/24, named MGMT.

Every 802.1Q trunk needs a native VLAN. Leaving it as the default VLAN 1 is a recognised VLAN-hopping exposure. Separately, the nine switches need an IP address of their own or SSH cannot reach them at all. VLAN 99 solves both problems with one subnet. It is carried on every trunk as the native VLAN and routed by Gi0/0.99, which uses the native keyword so that untagged frames are handled correctly.

VLAN 999, named PARKING-UNUSED.

An unrouted dead-end VLAN. Every unused switch port is placed in it and then shut down, so an unattended port in a corridor gives an attacker nothing.

The eight departmental VLANs and their subnets are exactly as specified in the brief. Nothing was changed.

## 5.3 DHCP on the router, not on a server

All seven pools live on R1-CORE. One authority, one place to look, and it keeps working when a server is powered off. It also means that show ip dhcp binding on the router is a single source of truth for all thirty-six leases.

A consequence worth knowing: every Packet Tracer Server has its own DHCP service, and if one is left switched on it races the router and hands out addresses from the wrong subnet. The setup document therefore turns DHCP explicitly off on all three servers.

## 5.4 Servers are static

Their addresses are referenced by the DNS A records, by both extended ACLs, and by the dns-server option in seven DHCP pools. A server that changed address on reboot would break all three at once.

# 6. Security Design

Security is layered rather than concentrated in one place. Nothing in this section blocks DHCP, DNS or HTTP, which was an explicit constraint in the brief.

| Control | Where applied | What it achieves |
| --- | --- | --- |
| enable secret | All 10 devices | Hashed privileged-mode password |
| service password-encryption | All 10 devices | Obscures remaining plaintext passwords |
| Console password, exec-timeout 5 | All 10 devices | Physical access control and idle logout |
| SSH version 2, transport input ssh | All 10 devices | Telnet disabled outright |
| access-class 10 in on VTY | All 10 devices | Management reachable only from 192.168.10.0/24 |
| ACL 110 inbound on Gi0/0.70 | R1-CORE | Student labs cannot reach Admin or Faculty |
| ACL 120 outbound on Gi0/0.80 | R1-CORE | Server VLAN accepts only DNS, HTTP, HTTPS, FTP, ICMP |
| Port security, max 1, sticky, restrict | All 39 access ports | One MAC per port, violations counted and dropped |
| spanning-tree bpduguard enable | All access ports | A rogue switch on a user port is err-disabled |
| VLAN 999 plus shutdown | All unused ports | No unattended live port anywhere |
| Native VLAN 99, not VLAN 1 | All 9 trunks | Removes the classic VLAN-hopping exposure |

## 6.1 The three access control lists in detail

ACL 10, standard, applied to the VTY lines with access-class 10 in.

Permits 192.168.10.0/24 only. Everything else is caught by the implicit deny. The practical effect is that SSH to any router or switch works from an Administration PC and is refused from everywhere else.

ACL 110, extended, applied inbound on Gi0/0.70.

Permits DHCP on UDP 67 and 68, DNS to the name server, and HTTP and HTTPS to the two web hosts. It then denies all traffic from the student labs into the Administration and Faculty subnets, and permits everything else.

The DHCP permits are not decorative. An inbound ACL on a subinterface also filters traffic addressed to the router itself, and a DHCP DISCOVER is a broadcast to UDP 67. Omitting those lines is a classic way to break address assignment for an entire department while believing the ACL only touches inter-VLAN traffic.

It is applied inbound because filtering as close to the source as possible drops the packet before the router spends any effort routing it.

ACL 120, extended, applied outbound on Gi0/0.80.

Permits full IP access from Administration, then DNS to .10, HTTP and HTTPS to .20, HTTP and FTP to .30, and ICMP to the whole server subnet so that ping tests still pass. It then denies everything else entering the server VLAN and permits all remaining traffic.

It is applied outbound because a single ACL on the way into the server VLAN protects the servers from all seven other VLANs at once, while leaving return traffic from the servers untouched. Applying it inbound instead would filter replies and break the network in a way that is genuinely confusing to diagnose.

# 7. Deliverables

Sixty files, all present in the repository and pushed to GitHub.

| File | Purpose |
| --- | --- |
| README.md | Entry point. Requirements, repo layout, VLAN summary, credentials, security summary, git workflow, design rationale |
| .gitignore | Excludes PT backups, .pka activity files, logs, OS clutter. Deliberately does not ignore .pkt |
| configs/R1-core-router.txt | Paste-ready CLI for the 2911, 340 lines in 8 numbered blocks |
| configs/SW-DIST.txt | Distribution switch: 10 VLANs, 9 trunks, STP root, management SVI |
| configs/SW-ADMIN.txt ... SW-SRV.txt | Eight access switches, one per department, same template with per-department values |
| docs/topology-diagram.txt | ASCII topology showing every device, every port and every link type |
| docs/device-list.md | Device list with models and quantities, plus the full VLAN table |
| docs/ip-addressing-table.md | Every interface in the campus with a duplicate-address audit |
| docs/cable-connection-list.md | All 48 cables, source port to destination port, with cable types |
| docs/pc-server-setup.md | Click-by-click GUI steps for the 36 PCs and 3 servers |
| docs/build-checklist.md | Ten phases from blank file to demonstrable network, plus a 12-minute demo run sheet |
| docs/testing-results.md | Eighteen tests with commands, locations and expected output, plus a recording sheet |
| docs/troubleshooting.md | Twelve realistic failures with symptom, cause, diagnostic command and fix |
| docs/portability-notes.md | Packet Tracer version traps and seven simulator limitations with workarounds |
| docs/laptop-setup.md | Cloning and running the project on a second machine |
| web/index.html | Campus portal page for SRV-WEB, listing all departments and services |
| web/portal.html | Application portal page for SRV-APP |
| packet-tracer/university-network.pkt | The built simulation: 49 devices, 48 links, all configs saved. Built on PT 9.0.1 |
| packet-tracer/README.md | How to open and verify the simulation, and the fallbacks for a version mismatch |
| paste/ (31 files + README) | Comment-stripped config chunks, split at the RSA prompt, one paste operation each |

Every configuration file is structured as numbered blocks with a header that states the device, model, role, port map and passwords, and with inline VERIFY comments giving the show command to run after each block. The files are paste-ready: they begin at enable and end at write memory.

# 8. Verification Built Into the Project

Eighteen tests are documented, each with the command, the device to run it on, and the expected output. The most diagnostically useful ones are:

| Test | Run on | Command | Expected |
| --- | --- | --- | --- |
| Same-VLAN ping | PC-CSE1 | ping 192.168.20.12 | Replies with TTL 128 |
| Cross-VLAN ping | PC-CSE1 | ping 192.168.30.11 | Replies with TTL 127 |
| ACL enforcement | PC-LAB1 | ping 192.168.10.11 | Destination host unreachable |
| Name resolution | PC-ECE1 | nslookup www.university.local | Resolves to 192.168.80.20 |
| End-to-end | any PC | ping www.university.local | Resolves then replies |
| Web portal | PC-LAB1 browser | http://www.university.local | Portal page renders |
| Routing table | R1-CORE | show ip route | Nine C routes and nine L routes |
| DHCP | R1-CORE | show ip dhcp binding | 36 bindings, none below .11 |
| Trunking | SW-DIST | show interfaces trunk | 9 trunks, native VLAN 99 on all |
| Port security | SW-LAB | show port-security | 8 ports, max 1, action Restrict |

The TTL distinction is the point worth understanding.

A same-VLAN ping returns TTL 128 because the packet never crossed a router. A cross-VLAN ping returns TTL 127, exactly one less, because it crossed precisely one router hop. A gateway ping returns TTL 255 because the reply originated on the router itself. Those three numbers tell you where a packet actually went, without needing any capture.

Two tests are designed to fail.

PC-LAB1 pinging the Administration and Faculty subnets must return Destination host unreachable. That is ACL 110 working. A student demonstrating this project should present those two failures as evidence, not apologise for them.

# 9. Session Chronology

What happened, in order, and why.

## 9.1 Design and authoring

The environment was checked first: Packet Tracer 9.0.1 was found installed, git 2.54 was available, and the Documents folder was identified as OneDrive-redirected. The folder R075458 was created and the twenty-three original files were written into it.

The eight access-switch configurations were generated from a single parameterised template so that all eight are structurally identical and differ only in hostname, VLAN, port count, management address and uplink port. This eliminates the copy-paste divergence that normally creeps into a set of near-identical device configs.

## 9.2 A consistency error found and corrected

A cross-file audit compared every uplink target, management address and access VLAN against the cabling list and the addressing table. It found one genuine inconsistency: the README claimed that access-class 10 in was applied to the VTY lines of every device, but the ACL had only been written into the router configuration. The nine switch configurations were amended to define ACL 10 and apply it to both VTY ranges, which brought the implementation into line with what the documentation promised.

## 9.3 The repository was moved out of OneDrive

The project was initially created under C:UserssohanOneDriveDocumentsR075458. A check confirmed that OneDrive was running and that Documents was redirected into it, which put the .git directory inside a continuously synced folder.

Two concrete problems with that:

- OneDrive can lock .git internals mid-operation, which corrupts the git index

- More importantly, if OneDrive already syncs Documents to the laptop, cloning the repository there would produce two copies of the same project, only one of which git is actually tracking

The project was therefore moved to C:UserssohanProjectsR075458, outside sync. Repository integrity was verified after the move with git fsck and a clean status check.

## 9.4 Git identity

The first two commits were made with an author address taken from session context rather than from the machine's configured git identity. This was caught before anything was pushed. Because the repository had no remote at that point, both commits were rewritten cleanly to the chosen identity, Chandhra Sekhar with the GitHub noreply address.

The noreply form was used deliberately. It attributes commits to the correct GitHub profile, with avatar and contribution graph, while keeping a real email address out of a public repository's permanent commit history where it would be scraped.

The identity was set repository-local rather than global, so the machine owner's own git identity is unaffected in every other project on the same machine.

## 9.5 Publication

The target repository, chandu242006/CN_proj, was found to exist, be public, and be empty. Empty mattered: had it been initialised with a README on GitHub, the two histories would have had no common ancestor and the first push would have been rejected.

A permissions check showed the signed-in CLI account had read access only. Two routes were available, adding the signed-in account as a collaborator, or signing the CLI in as the repository owner. The second was chosen. The interactive login was performed by the user, not automated, since it involves another person's credentials. The push then succeeded and was verified by comparing the remote HEAD against the local commit.

## 9.6 Build execution

The simulation was built by hand in the Packet Tracer GUI. Forty-nine devices were placed and renamed, forty-eight cables were run, and the ten device configurations were applied. Cabling was deliberately completed before any switch configuration was pasted, because port security uses sticky MAC learning and each port should see the correct device on its first frame.

Two checkpoints were used rather than testing only at the end. After the router configuration, the routing table was expected to list nine connected and nine local routes. After the distribution switch was configured, the router subinterfaces were re-checked and had moved to up and up, which is the first point at which the design can be said to work.

The servers were then given static addresses and services, the two portal pages were pasted into their HTTP index files, and the thirty-six PCs were switched to DHCP.

## 9.7 Paste tooling

Packet Tracer silently drops characters when a large block is pasted into a device CLI. The result is a configuration that looks correct on screen and does not work, which is a difficult failure to diagnose because nothing reports an error.

To remove that risk, the ten annotated configuration files were mechanically converted into thirty-one paste chunks in a new paste directory. Comments and blank lines were stripped, each chunk was limited to roughly seventy lines and cut only at top-level command boundaries, and every chunk was cut at exactly the line where the RSA key generation command interrupts a paste to ask for a key size.

Each chunk is then a single copy-and-paste with no scrolling or selection. The split was verified across all ten devices: every first chunk begins at enable, every RSA boundary lands correctly, and every final chunk ends at write memory. The annotated files in the configs folder remain the readable source of truth.

## 9.8 Publication and two authentication obstacles

The first obstacle was permission. The repository belongs to one GitHub account while the build machine was signed in as another, which held read access only. A push would have failed with a permission error that does not explain itself. The chosen resolution was to sign the CLI in as the repository owner, performed by the user rather than automated, since it involves another person's credentials.

The second obstacle was subtler and would have caused a confusing failure. Git and the GitHub CLI are separate programs with separate credential stores. The CLI was authenticated correctly, but Git was configured to use Git Credential Manager, which holds its own GitHub credential independently and could easily have supplied the wrong account. Pointing Git at the CLI token resolved it, so both now use the same active account.

The simulation file was then committed with the Packet Tracer version recorded in the commit message, and pushed. The push was verified by comparing the remote branch head against the local commit and confirming the file size on the remote.

# 10. Final Status

The project is complete and published. Every item in the original brief has been delivered, the simulation has been built and verified, and all sixty files are on GitHub.

| Deliverable | Status |
| --- | --- |
| Topology diagram | Complete |
| Device list and VLAN table | Complete |
| IP addressing table | Complete |
| Cable and connection list | Complete |
| Router configuration | Complete, applied and verified |
| Nine switch configurations | Complete, applied and verified |
| PC and server setup | Complete, applied and verified |
| Testing plan, eighteen tests | Complete |
| Troubleshooting, twelve symptoms | Complete |
| Build checklist | Complete |
| Portability notes and git workflow | Complete |
| Packet Tracer simulation file | Built on PT 9.0.1, committed and pushed |
| Repository published to GitHub | Complete, remote head verified |

The simulation file is self-contained. It carries the forty-nine devices, the forty-eight cables, all ten saved device configurations, the three servers with their static addresses and services, the five DNS records, both portal pages, the FTP user and every PC's DHCP setting. Opening it on another machine reproduces the finished network with no reconfiguration required.

## 10.1 Verification performed

The build was checked at each stage rather than only at the end.

- Router subinterfaces reached up and up once the distribution trunk came online

- Nine connected and nine local routes present in the routing table

- Nine trunks confirmed, with native VLAN 99 on every line

- Thirty-six DHCP bindings issued, none below the excluded range, none in the server subnet

- Name resolution and end-to-end reachability confirmed by pinging the web server by name

- Both intended ACL denials confirmed as failures, which is the design working as specified

- Port security demonstrated by moving a host between ports and observing the violation counter increment while the port remained in the secure-up state

- SSH confirmed reachable from the Administration subnet and refused from another subnet

# 11. What Remains

Nothing in the brief is outstanding. Four optional items remain, listed by how much they matter.

## 11.1 Verify the clone on the second machine

The repository has not yet been cloned and opened on the laptop. This is the last functional step and takes about ten minutes: clone into a folder outside any cloud-synced directory, open the simulation file, then check the router interface summary and ping the web server by name from any PC.

One hard prerequisite.

The file was saved by Packet Tracer 9.0.1. Compatibility runs one way only: a newer Packet Tracer opens older files, an older one never opens newer files, and the error message does not identify the version as the cause. If the laptop runs anything older, the file must be re-saved down a version from a machine that can already open it, using File then Save As and choosing a lower entry in the Save as type dropdown. That can only be done on the build machine, so it is worth doing before leaving it.

## 11.2 Fill in the test recording sheet

The testing document contains a twenty-row pass and fail table that is still blank. The tests were run during the build, but recording the results converts a test plan into test evidence, which is usually what is assessed.

## 11.3 Add screenshots

The repository contains no screenshots. The five most useful would be the topology canvas, the router interface summary, the trunk summary on the distribution switch, a PC browser showing the portal page, and the port security violation counter after the deliberate violation.

## 11.4 Commit a down-version simulation file

Saving a second copy in an older Packet Tracer format and committing it alongside the current one removes the version risk permanently, for any machine the project is ever opened on.

# 12. Known Limitations

Simulator constraints that shaped the design, rather than defects in it.

| Limitation | Consequence | Workaround adopted |
| --- | --- | --- |
| PT browser understands almost no CSS | Modern markup will not render | Portal pages written with table, font and bgcolor markup |
| PT Servers can run DHCP | A server left on races the router | DHCP explicitly turned off on all three servers |
| PT DNS has no forwarding or recursion | External names cannot resolve | Every tested name exists as an explicit A record |
| Router-on-a-Stick is a bandwidth hairpin | Inter-VLAN traffic crosses Gi0/0 twice | Accepted at this scale; SVI alternative documented |
| Sticky MAC is fragile against re-cabling | Moving a PC causes violations | Cable before configuring; recovery documented |
| Gigabit and FastEthernet do not interoperate | Mixed-speed link stays down | Switch links Fa-to-Fa, router link Gi-to-Gi |
| PT 8.0+ requires a Cisco login | Files will not open when signed out | Noted as a prerequisite in README and laptop guide |
| Newer .pkt cannot open in older PT | Silent incompatibility across machines | Save As down-version, plus full rebuild from configs/ |

The .pkt version trap deserves emphasis.

Packet Tracer file compatibility runs one way only. A newer installation opens older files; an older installation never opens newer ones, and the error message does not tell you which version created the file. This machine runs 9.0.1, so a file saved here will not open on a laptop running 8.2. The mitigations are to match versions, to use File then Save As and pick an older format, or to rebuild from the plain-text configurations. The third option is the reason configs/ exists at all: a version mismatch becomes an inconvenience rather than a loss of work.

# 13. Quick Reference

## 13.1 Lab credentials

These are teaching-lab values and are stored in a public repository. They must be changed before any of this is reused on real hardware.

| Where | Username | Password |
| --- | --- | --- |
| R1-CORE enable | - | Univ@Core#2025 |
| SW-DIST enable | - | Univ@Dist#2025 |
| Access switch enable | - | Univ@<Dept>#2025, e.g. Univ@Cse#2025 |
| Console, all devices | - | Cons@2025 |
| VTY fallback | - | Vty@2025 |
| SSH, all devices | netadmin | Adm1n@Univ#2025 |
| FTP on SRV-APP | student | Student@2025 |

## 13.2 Commands

Clone the repository:

git clone https://github.com/chandu242006/CN_proj.git

Before opening Packet Tracer, always:

git pull

After closing Packet Tracer:

git add -A

git commit -m "describe what changed"

git push

A .pkt is a binary blob that git cannot merge. If the same simulation is edited on two machines without pulling in between, git reports a conflict it has no way to resolve and one side's work is lost. Pull before opening, push after closing.