University Network with Multiple Subnets

Demonstration Guide

How to present this project in Cisco Packet Tracer

Project R075458   |   Cisco Packet Tracer 9.0.1   |   Approx. 15 minutes

# Part 1. What This Project Is

A university campus has eight departments that must share one physical network without sharing one flat network. If every machine sat in the same broadcast domain, a student laboratory PC could reach the Administration file server directly, one faulty machine could flood the entire campus with broadcasts, and there would be no place to enforce a policy.

This project solves that. Each department is placed in its own VLAN and its own IP subnet, the departments are connected by a single router that controls all traffic between them, and access control lists on that router decide which department may reach which.

## The design in one paragraph

Eight departmental VLANs, numbered 10 to 80, one per department. A ninth VLAN, 99, carries switch management and is the native VLAN on every trunk. A Cisco 2911 router provides the gateway for all of them using Router-on-a-Stick: nine 802.1Q subinterfaces on a single physical port. The router is also the DHCP server for the seven user VLANs and the enforcement point for the access control lists. Three servers in VLAN 80 provide DNS, a web portal and an FTP file service, and they are statically addressed.

## The numbers

| Item | Count |
| --- | --- |
| Departments / VLANs | 8 (plus management VLAN 99) |
| Routers | 1 x Cisco 2911 |
| Switches | 9 x Cisco 2960 (1 distribution, 8 access) |
| End devices | 36 PCs + 3 servers |
| Total devices | 49 |
| Cables | 48 (9 trunk, 39 access) |
| Subnets | 9 x /24 |
| DHCP pools | 7 (servers and management are static) |
| Access control lists | 3 |

## Departments and addressing

| VLAN | Department | Network | Gateway | Hosts |
| --- | --- | --- | --- | --- |
| 10 | Administration | 192.168.10.0/24 | 192.168.10.1 | 4 PCs |
| 20 | Computer Science | 192.168.20.0/24 | 192.168.20.1 | 6 PCs |
| 30 | Electronics | 192.168.30.0/24 | 192.168.30.1 | 6 PCs |
| 40 | Mechanical | 192.168.40.0/24 | 192.168.40.1 | 4 PCs |
| 50 | Library | 192.168.50.0/24 | 192.168.50.1 | 4 PCs |
| 60 | Faculty | 192.168.60.0/24 | 192.168.60.1 | 4 PCs |
| 70 | Student Labs | 192.168.70.0/24 | 192.168.70.1 | 8 PCs |
| 80 | Server Room | 192.168.80.0/24 | 192.168.80.1 | 3 servers |
| 99 | Management | 192.168.99.0/24 | 192.168.99.1 | 9 switch SVIs |

Servers: DNS at 192.168.80.10, Web at 192.168.80.20, Application and FTP at 192.168.80.30. The internal domain is university.local, with A records for www, dns, portal and ftp.

## How the three tiers connect

| Tier | Device | Role |
| --- | --- | --- |
| Core | R1-CORE (2911) | Gateway for all nine VLANs on 802.1Q subinterfaces. DHCP server. ACL enforcement. |
| Distribution | SW-DIST (2960) | Aggregates all eight access switches. Spanning-tree root. Single trunk to the router. |
| Access | 8 x 2960 | One switch per department. All user ports are access ports in that department's VLAN. |

There are no routed point-to-point links. Every packet between two departments travels up the trunk to the router and back down, which is what Router-on-a-Stick means.

## What the security does

| Control | Effect |
| --- | --- |
| ACL 110 on the Student Lab VLAN | Lab PCs cannot reach Administration or Faculty. They keep DHCP, DNS and web access. |
| ACL 120 on the Server VLAN | Servers accept only DNS, HTTP, HTTPS, FTP and ping. Administration keeps full access. |
| ACL 10 on the VTY lines | Remote management is possible only from the Administration subnet. |
| Port security on all access ports | One MAC address per port. A swapped machine is detected and its frames dropped. |
| Unused ports disabled | Every spare port is shut down and parked in a dead VLAN. |
| Native VLAN 99, not VLAN 1 | Removes the standard VLAN-hopping exposure. |

# Part 2. Before You Present

Do this five minutes before you are due to speak, not while the examiner is watching.

- Open Packet Tracer and sign in with your Cisco account. It will not open any file until you do.

- Open packet-tracer/university-network.pkt.

- Wait until every link light is green. From a cold start spanning tree takes about thirty seconds and the lights are amber until it settles. Do not begin while they are amber.

- Confirm the network is healthy with the single check below.

The one-command health check

On any PC, open Desktop then Command Prompt and run:

ping www.university.local

If the name resolves to 192.168.80.20 and you get replies, then DHCP, DNS, inter-VLAN routing and the server VLAN are all working. That single command covers most of what you are about to demonstrate.

Two habits that make the demo go smoothly

- Ignore the first ping. Packet Tracer almost always shows one Request timed out on the first attempt while ARP resolves. Say so out loud once, then judge every test by the replies that follow.

- Keep the CLI font readable. If you are presenting on a projector, widen the device window before you start rather than during.

Suggested order and timing

| Step | What you show | Minutes |
| --- | --- | --- |
| 1 | The topology and the three tiers | 1 |
| 2 | VLANs on a departmental switch | 1 |
| 3 | Trunks on the distribution switch | 1 |
| 4 | The router: subinterfaces and routing table | 2 |
| 5 | DHCP working, from both sides | 2 |
| 6 | Same-VLAN vs cross-VLAN ping, and the TTL story | 2 |
| 7 | DNS and the web portal | 2 |
| 8 | Access control lists blocking traffic | 2 |
| 9 | Port security catching a swapped machine | 2 |
| 10 | SSH allowed and refused | 1 |
| 11 | 802.1Q tagging in Simulation mode | 2 |

# Part 3. The Demonstration

## Step 1. The topology

Stay on the workspace. Point at each tier as you name it.

SAY:  This is a three-tier campus network. At the top is the core router, R1-CORE. Below it is the distribution switch, which every departmental switch connects to. At the bottom are eight access switches, one per department, with their PCs. Forty-nine devices and forty-eight cables in total.

Point at the single cable between R1-CORE and SW-DIST.

SAY:  This one link carries all nine VLANs. That is the Router-on-a-Stick design: instead of nine physical router ports, there is one physical port divided into nine logical subinterfaces.

## Step 2. VLANs on a departmental switch

Click SW-CSE, open the CLI tab, press Enter, then:

enable

Password: Univ@Cse#2025

show vlan brief

What to point out

- All ten VLANs exist on the switch, not just its own.

- Fa0/1 to Fa0/6 are listed under VLAN 20, which is Computer Science.

- Fa0/24 appears under no VLAN at all. That absence is how you prove it is a trunk port, because trunks belong to every VLAN rather than one.

- The spare ports sit in VLAN 999, a dead-end VLAN, and are shut down.

SAY:  Every switch knows the full VLAN list so that a trunk never has to drop a frame for a VLAN it does not recognise. The user ports here are locked to VLAN 20, and the uplink is a trunk, which is why it is not listed against any single VLAN.

## Step 3. Trunks on the distribution switch

Click SW-DIST, CLI tab:

enable

Password: Univ@Dist#2025

show interfaces trunk

What to point out

- Nine trunk ports: one uplink to the router and eight downlinks to the access switches.

- The Native vlan column reads 99 on every single line.

- The allowed list contains exactly the nine routed VLANs and nothing else.

SAY:  The native VLAN is set to 99 rather than the default VLAN 1. Leaving it at 1 is a known VLAN-hopping weakness, and it also gives the switches somewhere to hold a management address. If these two ends disagreed on the native VLAN, that department would silently lose connectivity.

## Step 4. The router

Click R1-CORE, CLI tab:

enable

Password: Univ@Core#2025

show ip interface brief

What to point out

- GigabitEthernet0/0 is up but has no IP address. That is correct for the physical trunk port.

- Nine subinterfaces, Gi0/0.10 through Gi0/0.99, each up and up, each holding one gateway address.

- Gi0/1 and Gi0/2 are administratively down, because unused ports are disabled.

SAY:  The physical port carries no address. Each subinterface handles one VLAN's tagged traffic and owns that subnet's gateway. This is the whole of the inter-VLAN routing configuration.

Then:

show ip route

SAY:  Nine connected routes and nine local routes, and no routing protocol anywhere. Every subnet is directly attached to a subinterface, so the router already knows how to reach all of them. That is one advantage of this design at campus scale.

## Step 5. DHCP, from both sides

First from the client. Click PC-CSE1, Desktop, Command Prompt:

ipconfig /all

What to point out

- The address is in 192.168.20.0/24, so the PC got a lease from the correct pool for its VLAN.

- The default gateway is 192.168.20.1, the router subinterface.

- The DNS server is 192.168.80.10, handed out by the DHCP pool.

- The host part is .11 or above, never lower.

Now from the server side. Back on R1-CORE:

show ip dhcp binding

SAY:  Thirty-six leases, four to eight per department, and nothing below .11 in any subnet because the first ten addresses are reserved for gateways and printers. There are no leases at all in the server subnet, because the three servers are static by design. Their addresses are referenced by the DNS records and by the access lists, so a server that changed address on reboot would break both.

## Step 6. Same-VLAN and cross-VLAN, and the TTL story

This step proves routing is actually happening, using nothing but the TTL value.

First, a ping inside one VLAN.  On PC-CSE1:

ping 192.168.20.12

Replies come back with TTL=128.

Now a ping across VLANs.  Still on PC-CSE1:

ping 192.168.30.11

Replies come back with TTL=127.

SAY:  These two numbers are the proof. Inside one VLAN the packet never passes a router, so the time-to-live is untouched at 128. Across VLANs it comes back as 127, exactly one less, because it crossed precisely one router hop. And if I ping the gateway itself the reply is TTL 255, because it came from the router rather than from a host behind it.

Optionally, show the path directly:

tracert 192.168.30.11

Two hops. Hop one is the local gateway, hop two is the destination.

## Step 7. DNS and the web portal

On any PC, Command Prompt:

nslookup www.university.local

It resolves to 192.168.80.20.

Then:

ping www.university.local

SAY:  The PC never received that address by hand. DHCP told it which DNS server to use, the DNS server resolved the name, and the router carried the traffic across two VLANs to reach it. That one command exercises the whole stack.

Now the visual finish. On the same PC, Desktop, then Web Browser. Type:

http://www.university.local

The campus portal loads, listing every department, its VLAN, its subnet and its gateway.

Then try the second server:

http://portal.university.local

SAY:  Two separate servers, two separate pages, both reached by name rather than by address.

## Step 8. Access control lists blocking traffic

This is where the design stops being just connectivity and becomes policy. Two of these tests are meant to FAIL. Say so before you run them.

SAY:  The next two pings are supposed to fail. That is the security policy working, not a fault in the network.

On PC-LAB1, a student laboratory machine, Command Prompt:

ping 192.168.10.11

Result: Destination host unreachable, and note that the reply comes from 192.168.70.1, the router itself.

ping 192.168.60.11

Same result.

Now, from the same PC, show what still works:

ping 192.168.20.11

ping www.university.local

Both succeed. Then open the Web Browser on PC-LAB1 and load the portal. It works.

SAY:  The student lab is blocked from Administration and from Faculty, but it keeps DHCP, DNS, the web portal and access to the other academic departments. That selectivity is the point. A blanket block would have been easy; the access list permits the services first and denies only the two sensitive subnets.

Finish by showing the counters. On R1-CORE:

show access-lists

SAY:  The match counters on the two deny lines have incremented. That is numeric proof the rule fired, rather than an assumption that it did.

## Step 9. Port security catching a swapped machine

The most convincing live moment in the demonstration, because you cause the failure yourself.

Set it up

- Select the Delete tool from the toolbar.

- Delete the cable between PC-LAB1 and SW-LAB Fa0/1.

- Delete the cable between PC-LAB2 and SW-LAB Fa0/2.

- Press Escape to leave the Delete tool.

- Using a Copper Straight-Through cable, connect PC-LAB2 to SW-LAB Fa0/1 - the port that used to belong to PC-LAB1.

Trigger it

On PC-LAB2, Command Prompt:

ping 192.168.70.1

It fails, even though the link light is green.

Show why

On SW-LAB, CLI, after enable with password Univ@Lab#2025:

show port-security interface FastEthernet0/1

Security Violation Count has become 1, Last Source Address shows PC-LAB2's MAC, and Port Status still reads Secure-up.

SAY:  The port had learned PC-LAB1's MAC address and pinned it. A different machine on that port is dropped and counted. Notice the port is still Secure-up, not disabled. That is what violation restrict means. The alternative, violation shutdown, would have disabled the port completely and needed an administrator to re-enable it.

Put it back

- Delete the PC-LAB2 to Fa0/1 cable.

- Reconnect PC-LAB1 to Fa0/1 and PC-LAB2 to Fa0/2 with straight-through cables.

- On PC-LAB1, ping 192.168.70.1 - it works again, because its MAC is the one the port trusts.

## Step 10. SSH allowed and refused

On PC-ADM1, an Administration machine, Command Prompt:

ssh -l netadmin 192.168.10.1

Password: Adm1n@Univ#2025. The banner appears and you reach the router prompt.

Now the same command from PC-CSE1, which is not in Administration:

ssh -l netadmin 192.168.10.1

Connection refused.

SAY:  Remote management is restricted to the Administration subnet by an access list on the VTY lines. Telnet is not merely unused, it is disabled outright, so the only way in is SSH from an authorised subnet.

## Step 11. Watching the VLAN tag in Simulation mode

If time allows, finish here. It makes the invisible visible.

- Switch from Realtime to Simulation, bottom right of the window.

- Click Edit Filters, clear everything, then tick only ICMP and ARP.

- From PC-CSE1, ping 192.168.30.11.

- Click Capture / Forward repeatedly and follow the packet.

- Click any packet and open Inbound PDU Details to see the 802.1Q header.

| Hop | What happens to the frame |
| --- | --- |
| PC to its access switch | Untagged - it is an access port |
| Access switch to SW-DIST | The switch adds an 802.1Q tag for VLAN 20 |
| SW-DIST to the router | Still tagged VLAN 20 |
| Inside the router | Routed from subinterface .20 to subinterface .30 |
| Router back down | Now tagged VLAN 30 |
| Access switch to PC | Tag stripped - access port again |

SAY:  This is Router-on-a-Stick shown rather than described. The tag is added at the access switch, changes VLAN inside the router, and is removed before the frame reaches the destination PC. Remember to switch back to Realtime afterwards.

# Part 4. Closing and Likely Questions

## How to close

SAY:  To summarise: eight departments, eight VLANs, eight subnets, one router providing all the gateways on a single physical port. Addresses are handed out automatically, names resolve internally, the web portal works, and the traffic between departments is filtered rather than merely forwarded. Every device configuration is also kept as a plain text file, so the whole network can be rebuilt by hand if the simulation file will not open.

## Questions you should expect

| Question | Short answer |
| --- | --- |
| Why Router-on-a-Stick and not a Layer-3 switch? | Simpler to build and to read, and it makes the 802.1Q tagging visible in Simulation mode. The trade-off is that every inter-VLAN packet crosses the same link twice. At 49 devices that is irrelevant; at several thousand users a Layer-3 switch would be correct. |
| Why is there a VLAN 99 when the brief asked for eight? | Every trunk needs a native VLAN, and leaving it as VLAN 1 is a VLAN-hopping exposure. The switches also need an address for SSH. VLAN 99 solves both. The eight departmental VLANs are unchanged. |
| Why put DHCP on the router instead of a server? | One authority and one place to check. It also keeps working if a server is switched off, and show ip dhcp binding becomes a single source of truth for all 36 leases. |
| Why are the servers static? | Their addresses are referenced by the DNS records, by both access lists and by the dns-server option in seven DHCP pools. An address change on reboot would break all three. |
| What happens if someone plugs a laptop into a spare port? | Nothing. Every unused port is shut down and parked in VLAN 999, which is not routed anywhere. |
| Why is the ACL on the lab VLAN inbound but the server one outbound? | Inbound on the lab subinterface drops the packet closest to its source, before the router does any work. Outbound on the server subinterface lets one access list protect the servers from all seven other VLANs at once, while leaving their replies untouched. |
| Could a student change their IP to get around the block? | Changing the address does not help, because the access list matches on the source subnet and the machine is physically in VLAN 70. It would also break their own connectivity, and port security limits each port to one MAC address. |

## If something fails during the demonstration

Stay calm and diagnose out loud - working through it methodically usually reads better than a demo where nothing went wrong.

| Symptom | First thing to check |
| --- | --- |
| A PC has no address | Was it left on Static? Desktop, IP Configuration, select DHCP. |
| One department cannot reach anything | show interfaces trunk on both ends of its uplink, and compare the native VLAN. |
| Names do not resolve | On the DNS server, confirm the service is On and the default gateway is 192.168.80.1. |
| The portal page will not load | Confirm HTTP is On for that server. Try the IP address first: if the address works and the name does not, it is DNS, not the web service. |
| A port went dead after you moved a cable | Port security. show port-security interface, then put the original machine back. |
| Everything is slow or flapping | Spanning tree is reconverging. Wait thirty seconds before touching anything else. |

The repository contains a full troubleshooting guide covering twelve failures with symptoms, causes, diagnostic commands and fixes, in docs/troubleshooting.md.

## Passwords you will need

| Device | Password |
| --- | --- |
| R1-CORE enable | Univ@Core#2025 |
| SW-DIST enable | Univ@Dist#2025 |
| Access switches enable | Univ@ plus department plus #2025, for example Univ@Cse#2025 |
| SSH login | netadmin / Adm1n@Univ#2025 |
| FTP on the application server | student / Student@2025 |